EEC-IV Wiring Information

Pin No	Wire Colour	Purpose	Voltage

1	 	Not Used

2	 	Not used

3	 	Not Used

4	 	Not Used

5	 	Not Used

6	 	Not Used

7	 	Engine Coolant Temp	20*C-3.1V, 40*C-2.2V, 100*C-0.5V

8	 	Not Used

9	 	Not Used

10	 	Airconditioning Clutch On	0V Aircon Off, 12V Aircon On

11	 	Not Used

12	 	Not Used

13	 	Not Used

14	 	Not Used

15	 	Not Used

16	 	Ground	0V

17	 	Self Test Output (STO)	PWM signal 0V-12V

18	 	Not Used

19	 	Not Used

20	 	Case Ground	0V

21	 	Idle Speed Control	+8V to +11V Engine Running

22	 	Fuel Pump Control	+12V Ignition On

23	 	Knock Sensor (Unleaded Only)	+3V Engine Running

24	 	Not Used

25	 	Intake Air Temperature Sensor	0*C 3.9V, 20*C 3,1V, 40*C 2.2V

26	 	Sensor Reference Voltage	+5V

27	 	Air Flow Sensor (Unleaded Only)	Variable Voltage

28	 	Not Used

29	 	Not Used

30	 	Transmission Position	Neutral 0V-0.1V, In gear 5Volts

31	 	Canister Purge	8V-10V with Engine Running

32	 	Not Used

33	 	EGR Solenoid (Unleaded)	Voltage Pulse @ 1800 RPM

34	 	Data Output Link	PWM Data

35	 	Canister Purge Solenoid (Unleaded)	Voltage Pulse when Engine Running

35	 	EGR Vent Solenoid (leaded)	Voltage Pulse when Engine Running @ 1800RPM

36	 	SPOUT (Spark Output) Ignition	7V average with Engine Running

37	 	Ignition Power	12V with Ignition On

38	 	Not Used

39	 	Not Used

40	 	Ground	0Volts

41	 	Not Used

42	 	Not Used

43	 	Air Flow Sensor (Leaded Only)	Variable Voltage

44	 	Not Used

45	 	Not Used

46	 	Signal Ground	0Volts

47	 	Throttle Position Sensor	0.7V closed, 4.5V WOT (Wide Open)

48	 	Self Test Input	0Volts for Self Test, 12V Normal operation

49	 	Not Used

50	 	Not Used

51	 	Not Used

52	 	EGR Vent Solenoid (Unleaded)	Voltage Pulse @ 1800 RPM

53	 	Not Used

54	 	EGR Vent Solenoid (leaded)	Voltage Pulse @ 1800 RPM

55	 	Not Used

56	 	PIP Profile Ignition Pick Up	0V - 12V on Ignition PIP

57	 	Ignition Power	0Volts Off, 12V Ignition On

58	 	Fuel Injection Ignition Bank 1,3,5	PWM On Fuel Injection

59	 	Fuel Injection Ignition Bank 2,4,6	0Volts Off, 12V On

60	 	Ground	0Volts